TO NOTE the Report and CONSIDER purchase of Bark for the Play Area

Background

Following the Council meeting of 11 March, I have carried out an extensive survey of the play area at Jubilee Field in respect of the likely effectiveness of the bark "safety surface" in both areas. The quality and depth of the surface was subject of comment in the report in the annual inspection carried out in September 2020. Reference was made to the maintenance of the bark requiring regular "forking over" to retain its effectiveness and furthermore made reference to the "national standard" of 300mm depth. In the course of carrying out the regular weekly inspections of the play areas the following was observed.

i.	The bark is compressed, due to foot-tread, weather has reduced the lower levels of bark to a much smaller component size.

ii.	The standards for ground cover within the "falls area" of equipment are insufficient and do not meet the standards quoted.

The figures quoted by test sources in respect of the recommended thickness of bark are:-	Fall height  5m  -  300mm

Fall height  4m  -  200mm

Fall height  1.5m  -  100mm

The areas identified around the multi-frame / slide where the depth of bark was found to be approximately 70mm, but, the fall height is between 1.5m at the top of the slide and 2.3m from the top of the chain climbing net. This requires an additional 100mm over the affected area would require approximately 2cu.m of bark allowing for further compaction.

There is adequate depth of bark at the infant/junior swing end of the park. Directly under the junior swings would benefit from some additional material. These could benefit from an additional 0.5 - 0.75cu.m of bark

Suitable play area bark is available from various sources as bulk bags from approximately £90 upwards (1cu.m. / 1000ltr) or in handle-able 100ltr. bags from DIY such as B&Q / Wickes at approximately the same cost.

Recommendation:

Purchase 3 Cu.m of suitable Bark (delivered in quantity as the handyman sees fit to handle.)  cost to be approved up to £350 from line 4131 play equipment maintenance.